|  | Simulation of LOX Tank Date: Thursday, January 15, 2026 Designer: Solidworks Study name: Static 1 with Thermal Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 6 Interaction Information 6 Mesh information 7 Sensor Details 8 Resultant Forces 9 Beams 9 Study Results 10 Conclusion 12. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: LOX Tank Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line12; Solid Body; Mass:4.45856 kg Volume:0.00165132 m^3 Density:2,700 kg/m^3 Weight:43.6939 N; C:\Users\starw\OneDrive\Documents\Capstone\liquid-KeroLOX-2026\CAD\Propellant Tanks\LOX-Tank-Assem\LOX Tank.SLDPRT Jan 13 18:39:44 2026.

# Study Properties Study name; Static 1 with Thermal. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; On. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\starw\OneDrive\Documents\Capstone\liquid-KeroLOX-2026\CAD\Propellant Tanks\LOX-Tank-Assem).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75e+08 N/m^2. Tensile strength:; 3.1e+08 N/m^2. Elastic modulus:; 6.9e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,700 kg/m^3. Shear modulus:; 2.6e+10 N/m^2. Thermal expansion coefficient:; 2.4e-05 /Kelvin.; SolidBody 1(Split Line12)(LOX Tank). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Roller/Slider-5; Entities:; 5 face(s). Type:; Roller/Slider.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.110168; 395.361; 0.884583; 395.362. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 24 face(s), 1 plane(s). Reference:; Front Plane. Type:; Apply force. Values:; ---, 17,000, --- lbf.. Force-2; Entities:; 24 face(s), 1 plane(s). Reference:; Front Plane. Type:; Apply force. Values:; ---, -17,000, --- lbf.. Temperature-1; Entities:; 103 face(s). Temperature:; -300 Fahrenheit.. Gravity-1; Reference:; Top Plane. Values:; 0 0 -88.29. Units:; m/s^2.. Pressure-2; Entities:; 1 face(s). Type:; Normal to selected face. Value:; 600. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Interaction Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 0.806082 in. Minimum element size; 0.0759627 in. Mesh Quality; High. ## Mesh information - Details Total Nodes; 98190. Total Elements; 45304. Maximum Aspect Ratio; 10.61. % of elements with Aspect Ratio < 3; 31.2. Percentage of elements with Aspect Ratio > 10; 0.00441. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:10. Computer name:; CHARLES. ## Mesh Control Information: Mesh Control Name; Mesh Control Image; Mesh Control Details. Control-2; Entities:; 1 face(s). Units:; in. Size:; 0.236787. Ratio:; 0.236787.. ## Mesh Quality Plots Name; Type; Min; Max. Quality1; Mesh; -; -. LOX Tank-Static 1 with Thermal-Quality-Quality1.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.110168; 395.361; 0.884583; 395.362. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0; 0; 0; 0. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.406e+03psi Node: 74080; 9.148e+04psi Node: 82273. LOX Tank-Static 1 with Thermal-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 1.167e-03mm Node: 54863; 1.517e+00mm Node: 25728. LOX Tank-Static 1 with Thermal-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 5.595e-05 Element: 1372; 4.732e-03 Element: 2599. LOX Tank-Static 1 with Thermal-Strain-Strain1.

# Conclusion